Design Philosophy	Faithfully replicate the home den environment while aggressively trying to achieve the maximum utilization of a very small confined space such that the result is a room that looks like mom’s den but feels spacious and comfortable enough to live and sleep in (or even better thrive in).

Design Restrictions   There can be no permanent alteration of the room (i.e. knocking out walls, removing cabinets, etc…) The room must be restored to its original state upon the end of occupancy (take before and after pictures of room to ensure).

# Primary Renovation Tasks

- Installation of woodgrain paneling and moulding on all walls (except kitchenette area).  Paneling must be secured using finishing nails or staple gun (no adhesive). Base moulding to be used on floor;  Quarter Circle (Base Shoe) moulding to be used on inside corners; Bead moulding to be used to frame call box (on adjoining south mini wall proximal to kitchenette), fire alarm (on east bed wall) door, heat/ac unit (on north wall), room door, and patio door;  Outside Corner moulding to be used on outside corners, window border, and outer closet frame;  Bed moulding used on ceiling and to separate paneling from mirror glass tile on specified walls (see next item).

- Installation of 12” x 12” mirror glass tiles to occupy upper four feet (semi wall) of 1) east bed wall and adjoining south mini wall (between kitchenette and north wall) 2) south mini wall (adjoining to west wall) and  3) south wall (between closet and room door).  Mirror tile may be secured to woodgrain paneling using strong adhesive with bed moulding as border on bottom; ~80 total tiles needed (see schematics for specific numbers of whole and partial glass tiles needed)

- Design, construction, and installation of wall unit on west wall.  Design should accommodate 19” TV/ VCR unit while providing suitable storage space for clothes and shelf space for pictures and decorations (TV dimensions: 19” width, 19” vertical length, 19” depth). Wall unit length will be ~100” out of 151” west wall in room leaving some space available for a small chair to be placed in the northwest corner of room (CALIFORNIA CLOSETS)

- Installation of carpet (style-watch hill; color-coffee bean) and thick plushy stain resistant padding. (HOME DEPOT – CARPET PROS)

# Secondary Renovation Task

- Installation of safety grab bars (See accompanying drawing for placement locations in room).

10” bar on small west wall adjoining to closet

42” bar on south wall proximal to closet

36” bar on south hallway wall outside room (between main door and room door

36” bar on north hallway wall outside room (between neighbors door and bathroom door)

36” bar on east hallway wall (between front door and kitchenette)

18” bar on north doorway mini wall.

- Install aromatic cedar paneling inside closet

- Install customized Closetmaid Shelftrack system in closet (See Closetmaid doc)

- Installation of towel rack, paper towel holder, and towel ring in kitchenette

- Installation of wall mounted light above bed

- Installation of decorative hanging light with cord/ chain swag

- Installation of two folding (accordion style) closet doors (double door design)

- Installation of fluorescent light in closet.  Will require installation of wiring and grounded outlet connected to power source located on external south wall of closet (must go through inner closet wall to tap into);  will also require installation of light switch on location proximal to closet door.

- Installation of headrail (must be cut to ~50”) and vertical blinds over window.  Blinds must be cut to 53” length (~15 individual blinds needed).

- Installation of 12”x12” adhesive vinyl tile on all cabinet shelves and drawers in kitchenette.

- Installation of “under the cabinet” style microwave underneath kitchenette cabinet.  Must remove lower front cabinet wood piece (put protective covering over exposed rough side of shelf) and back fluorescent utility light in order for microwave to fit (microwave dimensions – length 19” depth 11 3/4” height 11 1/4”).  Possibility of tapping into electrical wiring from removed utility light for power source.

- Installation of two small utility lights underneath cabinet in kitchenette (on either side of the microwave). Possibility of tapping into electrical wiring from removed utility light for power source or may need to install additional outlet to plug lights into.

- Installation of eight decorative hooks in ceiling to hang plant and light from.  (See accompanying drawing for placement locations in room).

- Installation of pre-assembled vanity cabinet on south wall of bathroom above the toilet (wall space – length 50” width 32”)

- Installation of custom wood shutters on inside patio door (HOME DEPOT – ALL WOOD SHUTTERS)

- Installation of retractable screen door on outside patio door.

- Install two-tiered sliding cabinet shelf in lower right kitchenette cabinet

- Install two-tiered racks behind sink, on face of lower right cabinet, and on side of upper cabinet in kitchenette

- Adjust main outer door between rooms 261L and 261R to open outward into facility hallway (Must research fire and building codes)

# Door Decoration Project

- Ornate door decoration for outer room door incorporating mom’s name, picture, and  life story.  Inner room door to have mural of library books.  (See document)

- Possibility of decorative wreath on outer hallway door (attach to door skin first)

- Possibility of door murals on inner hallway door and/ or bathroom door (attach to door skin first)

# Furniture

- Twin size chest bed & mattress with bookcase headboard and storage underneath.  Must fit in 83”x48”  space bordered by adjoining south mini wall, east bed wall, and north wall (DESIGN CENTER)

- Hanging light* (possibly get from Home Depot)

- Corner shelf organizer*

- Chair (padded, rocking, plushy).  Must fit in ~50” space between window and wall unit in northwest corner on room.

# Accessories

- bed - sheets x 3 (possibly rubber), electric heating pad*, eggcrate, blanket x 2, comforter

- decorative anti-slip pads for shower

- Sylvania 19” Color TV/ VCR Unit Model No. 44764 SRC 2419 (Sears)

- large floor plant(s)* and hanging plants

- pictures* and framed certificates*

- decorative items from living room and dining room*

- small cd stereo* & tape player*

- macramé hanging plant holders*

- Tuscany Collection Solid Wood Hangers (Home Depot) (or 99 cents only store plastic tube hangers)

- Stack & Expand Shoe Rack Model No. 8111 (Home Depot)

- Over the Door Shoe Rack Model No. 8040 (Home Depot)

- Tie & Belt Rack Model No.  8051 (Home Depot)

- Decorative Light Switch cover for closet light – Amerelle Leaf Expressions – Resin (Home Depot); Get matching double light switch for east hallway wall

- Photo memory board (w/ selected pictures) (Aaron Brothers)

- Decorative bathroom shower curtain

- Waste basket to put under sink

# Notes

- Take before, during, and after pictures of room

- Draw sketches of room to 1/4 scale using graph paper

- Carry tape measure at all times

*These items may potentially be brought from mom’s house